LXLC-350~600(mm)

Removable element woltman cold water meter

Application:

--Measuring the total volume of cold water passing through the pipeline.

-NOTE:This type of water meter can be used for a reading transmission system as equipped with pluse.

Feature:

-- Dry-dial, Magnetic drive

----Vacuum sealed register ensures the dial keep free from fog and Keep the reading clear in a long term service;

--Removable element unit and strong universality, easy installation and maintenance,

--Measuing high flow rate with low head loss.

-- Remote transmission device can be added upon request,such as Reed Switch or Hall, Pulse Output:100000L/P、10000L/P.

--Technical data conform to international standard ISO 4064.

Working condition

Water temperature: T30 T50

Water pressure class: MAP16  MAP10

Pressure-loss Class: △p63

Accuracy:

(1) From minimum flow-rate(Q1) inclusive to transitional flow-rate(Q2) exclusive±5%.

(2) From transitional flow-rate(Q2) to overload flow-rate(Q4):±2%;

### MAIN TECHNICAL SPECIFICATIONS

| 型号 Type | 公称口径Size | 计量等级Class | 过载流量 Q4 Overload Flow | 常用流量 Q3 Permanent Flow | 分界流量 Q2 Transitional Flow | 最小流量 Q1 Min.Flow |  |  | 最小读数 Min. Reading | 最大读数 Max. Reading |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | m3/h |  |  |  |  |  | m3 |  |
| LXLC-350 | 14〞 | R50 | 1750 | 1400 | 44.8 |  | 28 |  | 0.2 | 999,999,999 |
| LXLC-400 | 16〞 |  | 2000 | 1600 | 51.2 |  | 32 |  | 0.2 | 999,999,999 |
| LXLC-450 | 18〞 |  | 2000 | 1600 | 51.2 |  | 32 |  | 0.2 | 999,999,999 |
| LXLC-500 | 20〞 |  | 3125 | 2500 | 80 |  | 50 |  | 0.2 | 999,999,999 |
| LXLC-600 | 24〞 |  | 5000 | 4000 | 128 |  | 80 | 0.2 |  | 999,999,999 |

### DIMENSIONS

外形尺寸

| 型号 Type | 公称口径Size |  | 长 Length |  | 高 Height | 连接法兰Connecting flange |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | φD1 Outside diameter | φD2 Bolt circle diameter | Connecting Bolts(n-M) | Working pressure (MPa) |
|  | mm |  |  |  |  |  |  |  |  |
| LXLC-350 | 14〞 | 500 |  | 590 |  | 505 | 460 | 16-M20 | 1.0 |
|  |  |  |  |  |  | 520 | 470 | 16-M24 | 1.6 |
| LXLC-400 | 16〞 | 600 |  | 660 |  | 565 | 515 | 16-M24 | 1.0 |
|  |  |  |  |  |  | 580 | 525 | 16-M27 | 1.6 |
| LXLC-450 | 18〞 | 600 |  | 700 |  | 615 | 565 | 20-M24 | 1.0 |
|  |  |  |  |  |  | 640 | 585 | 20-M27 | 1.6 |
| LXLC-500 | 20〞 | 800 |  | 760 |  | 670 | 620 | 20-M24 | 1.0 |
|  |  |  |  |  |  | 715 | 650 | 20-M30 | 1.6 |
| LXLC-600 | 24〞 | 800 |  | 880 |  | 780 | 725 | 20-M27 | 1.0 |
|  |  |  |  |  |  | 840 | 770 | 20-M33 | 1.6 |

NOTE: The flange dimension conforms to ISO7005-2:1988 standard. Order for products of special requirements is also accepted.

| LXLC-350-500 REMOVABLE ELEMENT WOLTMAN WATER METER |  |  |  |
| --- | --- | --- | --- |
| Code | Description | Material | QTY |
| 1 | Screw For Sealing | Stainless Steel | 1 |
| 2 | Cover With Lid | Assembly | 1 |
| 3 | Sealed Register | Assembly | 1 |
| 4 | Screw M16×45 for sealing | Stainless Steel | 1 |
| 5 | Fixed plate | PPO | 1 |
| 6 | Adjusting Nut | Brass | 1 |
| 7 | Flange Cover | Cast Iron | 1 |
| 8 | O-ring | Silicon Rubber | 1 |
| 9 | Bush | Brass | 1 |
| 10 | Gasket | Synthetic Rubber | 1 |
| 11 | Adjusting Lever | Brass | 1 |
| 12 | Connecting Lever | Brass | 1 |
| 13 | Adjusting Plate | POM OR MPPO | 1 |
| 14 | Support and gear | Brass and pom | 2 |
| 15 | Turbine Component | Assembly | 1 |
| 16 | Rectifier | ABS OR MPPO | 1 |
| 17 | Screw | Brass | 1 |
| 18 | O-ring | Synthetic Rubber | 1 |
| 19 | Body | Cast Iron | 1 |
| 20 | Screw For Sealing | Stainless Steel | 2 |
| 21 | Screw For Sealing M16X45 | Stainless Steel | 13 |
| 22 | plate | Stainless Steel | 14 |
| 23 | Fixing screw | Stainless Steel | 4 |
| 24 | rings | Stainless Steel | 2 |
| 25 | Support block | Cast Iron | 2 |
| 26 | Upper Bearing Plate | Brass | 1 |
| 27 | Upper Bearing | Nylon With Graphite | 1 |
| 28 | Magnet Component | Component | 1 |
| 29 | casing | Stainless Steel | 1 |
| 30 | Transmission Shaft | Stainless Steel | 1 |
| 31 | Support | Nylon | 1 |
| 32 | Turbine Shaft | Tungsten Steel | 1 |
| 33 | Screw | Brass | 1 |
| 34 | Nut | Brass | 1 |
| 35 | rings | Stainless Steel | 2 |

----------------------------------------------------------------------------------------------------------------------

LIANYUNGANG HYTTEN METER CO.,LTD.

Add:No. 8 Hanguo Industrial Development Zone,Luoyang Village

Dunshang Town,Ganyu District, Lianyungang, Jiangsu, China

Tel:      ++86-518-85907891

Fax:     ++86-518-85907890             Postal code:   222000

E-mail: hytten@hyttenmeter.com

http://www.hyttenmeter.com    www.hyttenwatermeter.com