DN15-50 RF Card Smart Water Meter

DN15-50射频卡智能水表

Application:

--Measuring the total volume of cold(hot) water passing through the pipeline.

-NOTE:This type of water meter can be used for a reading transmission system as equipped with a built-in-sensor

Feature:

--RF water meter uses pre-paid way to achieve new management mode which is “Pay first, then use”

--Show the different price level of the water, one card can used for several meters

--Double reading dispay: mechanical and electronic

--Unique battery protection

--Stable performance, wide application

--Low power consumption and long working life

--Vacuum sealed electronic parts IP68

--Anti-Magnetic function

Working condition:

Water temperature: T30 T50 T90

Water pressure class: MAP16  MAP10

Installation environment: Class B

Electromagnetic environment: Class E1

Accurancy:

(1) From minimum flow-rate(Q1) inclusive to transitional flow-rate(Q2) exclusive±5%.

(2) From transitional flow-rate(Q2) to overload flow-rate(Q4):±2%(Hot water meter:; ±3%)

- MAIN TECHNICAL SPECIFICATIONS

| Type | Size | Class | Q4 Overload Flow | Q3 Permanent Flow | Q2t Transitional Flow | Q1 Minimum Flow | Min. Reading | Max. Reading |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | m3/h |  |  |  | m3 |  |
| LXSG-15/RF | ½" | R80 (R100) | 3.125 | 2.5 | 0.05 | 0.03125 | 0.00005 | 99,999 |
| LXSG-20/RF | ¾" |  | 5 | 4 | 0.08 | 00.05 | 0.00005 | 99,999 |
| LXSG-25/RF | 1" |  | 7.875 | 6.3 | 0.126 | 0.07875 | 0.00005 | 99,999 |
| LXSG-32/RF | 1¼" |  | 12.5 | 10 | 0.2 | 0.125 | 0.00005 | 99,999 |
| LXSG-40/RF | 1½" |  | 20 | 16 | 0.32 | 0.2 | 0.00005 | 99,999 |
| LXSG-50/RF | 2" |  | 31.25 | 25 | 0.5 | 0.3125 | 0.00005 | 99,999 |

- DIMENSIONS

| Type | Size | L Length | H Height | B Width | Connecting Thread |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | mm |  |  |  | D | d |
| LXSG-15/RF | ½" | 165 | 120 | 86 | G3/4 | R1/2 |
| LXSG-20/RF | ¾" | 195 | 120 | 90 | G1 | R3/4 |
| LXSG-25/RF | 1" | 225 | 120 | 85 | G11/4 | R1 |
| LXSG-32/RF | 1¼" | 328 | 210 | 99 | G11/2 | R11/4 |
| LXSG-40/RF | 1½" | 357 | 226 | 119 | G2 | R11/2 |
| LXSG-50/RF | 2" | 406 | 259 | 143 | G21/2 | R2 |

LIANYUNGANG HYTTEN METER CO.,LTD.

Add:No. 8 Hanguo Industrial Development Zone,Luoyang Village

Dunshang Town,Ganyu District, Lianyungang, Jiangsu, China

Tel:      ++86-518-85907891

Fax:     ++86-518-85907892             Postal code:   222000

E-mail: hytten@hyttenmeter.com

http://www.hyttenmeter.com    www.hyttenwatermeter.com