LXSG-15-50/IT LXLC-50-300/IT GPRS IOT Remote Transmission(Pre-paid Valve Control)Water meter.

Application:

--Measuring the total volume of cold(hot) water passing through the pipeline.

Feature

--Reading data through long distance wireless communication to avoid disturb residents.

--Effectively solve long time span by manual read data, And unable to read data problems at the same time to facilitate timely statistics of leakage.

--The data can be readed at any time, monitoring water data in real time and managing water efficiently.

--Remote monitoring function,convenient maintenance management.

--Ladder-like wate price.

--Use with valve, can realize prepaid,remote valve control.

--Urban network, realize intelligent water affairs.

--Complete in specifications:DN15-300mm

Working condition:

Water temperature: T30 T50

Water pressure class: MAP16  MAP10

Pressure-loss Class: △p63

Installation environment: Class B

Electromagnetic environment: Class E1

Accurancy:

(1) From minimum flow-rate(Q1) inclusive to transitional flow-rate(Q2) exclusive±5%.

(2) From transitional flow-rate(Q2) to overload flow-rate(Q4):±2%（Hot water meter: ±3%）

Technology Index

|  | Name | Parameter |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Min. | Model | Max. | Unit |  |
| 1 | Quiescen working current |  | 6 | 10 | uA |  |
| 2 | Dynamic working current |  |  | 2 | mA |  |
| 3 | Bluetooth communication working current |  | 30 |  | mA |  |
| 4 | GPRS Battery |  | +3.6 |  | V |  |
| 5 | GPRS low battery point | 2.6 |  | 2.8 | V |  |
| 6 | GPRS Normal Voltage | 2.5 |  |  | V |  |
| 7 | GPRS Communication times |  | 1000 |  | Times | Typical value: Battery Capacity>2.7AH |
| 8 | Operating Temperatuer | -10 |  | 50 | ℃ |  |
| 9 | Storage Temperature | -20 |  | 60 | ℃ |  |
| 10 | Humidity |  | 85% |  |  |  |
| 11 | Display time | 19 |  | 21 | S |  |
| 13 | Input Pulse width |  | 50 |  | ms |  |
| 14 | Bluebooth Baud Rate |  | 9600 |  | Bps |  |
| 15 | Motor Locked-rotor Current | 160 | 170 | 190 | mA |  |
| 16 | Waitting time when Motor drive overtime |  | 30 |  | S |  |
| 17 | Resistance of reed swith open circuit | 1 | ∞ |  | MΩ | Reed swith base on basic water meter |
| 18 | Effective Response time for Reed swith | 0.1 |  |  | S |  |
| 18 | Suit for Hall Singal |  |  |  |  |  |

LIANYUNGANG HYTTEN METER CO.,LTD.

Address: NO.8 Hanguo Industrial Development Zone,Luoyang village,Dunshang Town,Ganyu District, Lianyungang City.

Tel：0518-85907891         Fax：0518-85907892

Http：//www. hyttenmeter.com  E-mail：hytten@hyttenmeter.com