DN32-50mm DQF电动球阀

DQF Series DN32-50mm Electic Ball Valve

Application

Used for control the swith of water in the pipiline.

Note

The other connection ways will be made in further.

Feature

- Adopting Thread connection to connect with Romote Transmission convenient Installation.

- Actuators was sealed & installed Seperately,convenient for assembly & Maintenance.

- Control box and the valve body can be installed as monolithic or split type.

- Control box and Split type install individually on position tha easy to operate swiping card.

- The actuator rotate in forward and reversal direction by 90℃ to control the on-off swith of valve.

- Control install coil and moduless through full seal.

Working condition

- Use for clean tap water pipe, not suit for liquid that containing impurities and corrosive

- Water meter and the ball valve should avoid freezing,flooding,sunlight exposure and other bad environment.

- Actuator power supply by DC3.6V Lithium batteries.

- Operating temperature:0-90℃

- Water pressure:≤1.0MP

- The ball valve should not be blocked.

DIMENSION

| Type | Size | Dimension |  |  |  |  | Conecting Thread |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | L1 | H | h | B | R | D |
| DN32 | 1¼" | 101 | 113 | 158 | 120 | 98 | R1 1/2 | G1 1/2 |
| DN40 | 1½" | 110 | 122 | 178 | 140 | 98 | R2 | G2 |
| DN50 | 2" | 124 | 143 | 198 | 160 | 98 | R2 1/2 | G2 1/2 |

MAIN THCHNICAL SPECIFICATIONS

| Type | Size | Current parameter |  |  | Valve swiching time |
| --- | --- | --- | --- | --- | --- |
|  |  | No-load current | Mean load current | Max.load current |  |
| DN32 | 1¼" | 25 | 40(10 m3/h) | 75(10 m3/h) | 20-25 |
| DN40 | 1½" | 45 | 60(16 m3/h) | 110(16m3/h) | 50-55 |
| DN50 | 2" | 46 | 65(25 m3/h) | 120(25 m3/h) | 50-55 |

LIANYUNGANG HYTTEN METER CO.,LTD.

Add:No. 8 Hanguo Industrial Development Zone,Luoyang Village

Dunshang Town,Ganyu District, Lianyungang, Jiangsu, China

Tel:      ++86-518-85907891

Fax:     ++86-518-85907892             Postal code:   222000

E-mail: hytten@hyttenmeter.com

http://www.hyttenmeter.com    www.hyttenwatermeter.com