LXSG-15-40/ZD  光电直读

Dry-dial Photoelectric direct reading Water Meter

Application:

--Measuring the total volume of cold(hot) water passing through the pipeline.

-NOTE:This type of water meter can be used for a reading transmission system as equipped with a built-in-sensor

Feature:

--Reading data directly, realizes“zero”error basiclly

--Low power design.

--The intelligent electronic unit is completely sealed in the register to effectively avoid the erosion of external water and moisture.

--Meet electromagnetic compatibility design requirement for EMC,ESD,EMI etc electronic products.

--The fault diagnosis shutdown technology is used to guarantee the safety of the whole network.

--Provide M-BUS transmission.

Working condition:

Water temperature: T30 T50

Water pressure class: MAP16  MAP10

Pressure-loss Class: △p63

Installation environment: Class B

Electromagnetic environment: Class E1

Accurancy:

(1) From minimum flow-rate(Q1) inclusive to transitional flow-rate(Q2) exclusive±5%.

(2) From transitional flow-rate(Q2) to overload flow-rate(Q4):±2%(Hot water meter:; ±3%)

- Technical parameters

| 序号 | Communication mode | RS-485 | MBUS |
| --- | --- | --- | --- |
| 1 | working voltage | 9V～15V | 24V～42V |
| 2 | static current | ≤1mA | ≤2mA |
| 3 | wording current | ≤5mA | ≤10mA |
| 4 | Communication protocol | DL/T645、CJ/T 188（or customer requirements） |  |
| 5 | Communication rate | 1200/2400Baud |  |

- MAIN TECHNICAL SPECIFICATIONS(R50/100)

| Type | Size | Class | Qs Overload Flow | Qp Permanent Flow | Qt Transitional Flow | Qmin Min. Flow |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | m3/h |  |  |  |
| LXSG-15/ZD | ½" | R80 (R100) | 31.25 | 2.5 | 0.05 | 0.031 |
| LXSG-20/ZD | ¾" |  | 5 | 4 | 0.08 | 0.05 |
| LXSG-25/ZD | 1" |  | 7.875 | 6.3 | 0.126 | 0.079 |
| LXSG-32/ZD | 1¼" |  | 12.5 | 10 | 0.2 | 0.125 |
| LXSG-40/ZD | 1½" |  | 20 | 16 | 0.32 | 0.2 |

- Dimensions

- Installation Requirement

- The meter should be installed in horizontal position with the direction of the flow as in dicated by the arrow cast in the meter body with the register face upwards;

- Pipeline must be flushed before installation;

- The meter should be constantly full of water during operation;

- The meter must have 10 diameters straight pipe ahead of the meter and 5 diameters straight pipe after to insure proper flow through the meter.

DIMENSION

| Type | Size | L Length | L1 Length | B width |  | H Height | Connecting Thread | Connecting Wire Metal Hose |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mm |  |  |  |  |  | G | m |
| LXSG-15ZD | ½" | 259 | 165 | 90 | 125 |  | G3/4 | 1.0/0.5 |
| LXSG-20/ZD | ¾" | 299 | 195 | 90 | 125 |  | G1 |  |
| LXSG-25/ZD | 1" | 345 | 225 | 94 | 140 |  | G1 1/4 |  |
| LXSG-32/ZD | 1¼" | 354 | 230 | 94 | 140 |  | G1 1/2 |  |
| LXSG-40/ZD | 1½" | 373 | 245 | 125 | 162 |  | G2 |  |

Lead the definition

----------------------------------------------------------------------------------------------------------------------

LIANYUNGANG HYTTEN METER CO.,LTD.

Address: NO.8 Hanguo Industrial Development Zone,Luoyang village,Dunshang Town,Ganyu District, Lianyungang City.

Tel：0518-85907891         Fax：0518-85907892

Http：//www. hyttenmeter.com  E-mail：hytten@hyttenmeter.com